How to Run the Traffic Simulation Program

7 January 2011   Hans Camenzind

This Traffic Simulation Program is available free of charge.  You may run the program and distribute it to anyone, as long as you don't charge for it.  Commercial distribution is prohibited.

1.  Go to TrafficSim Files and transfer all the files into the same directory in your computer.

You should have:

TrafficSim.exe

400carsyield.csv

800carsnoyield.csv

Parameter Template.csv

2.  Double-click on TrafficSim.exe.  You will see the following screen:

The example loaded on start-up is trivial, just a few cars.  You can hit the ENTER key and run it or load one of the two examples provided (FILE, OPEN).  400carsyield.csv uses 400 a total of cars and parameters which reflect the European driving rules.  800carsnoyield.csv uses 800 cars with American driving habits.  For a full view, the display must be wide-screen.  Start the program by hitting ENTER.  The space bar will pause the program; hitting the space bar again will produce a single step and ENTER will make it continue.  You stop the simulation by clicking on the X in the top right corner of the screen.

Underneath the display is a small screen (a console window) which lists the momentary calculation.  To get numerical information click on a car in "pause" and go to the second screen.

3.  .csv examples, as well as Parameter Template, are comma-separated files (or Comma-Separated Values).  If you double click on them, they should come up in Excel and look like this:

If you want to write you own driver parameter you can use the Parameter Template.  The parameters are as follows:

"#"	Number of the group.  If there is no number, the line will be ignored

Z	Color of the group.  Some of the available colors are listed in the bottom line.

A	Desired speed (in mph).  This is how fast the cars in this group would like to go (but often don't)

B	Maximum acceleration (ft/sec/sec).   15 corresponds to 0 to 60mph in 6 seconds.

C	Minimum following distance, i.e. the minimum space in car-lengths to the car in front.  Please 	note that a distance in car-lengths is measured from center to center of two cars.  Thus a 1 car-	length is bumper-to-bumper; the minimum distance should be 2 car-lengths.

D	Delay after standstill (in seconds).  Standstill is defined as less than 10 mph.

E	Highest lane (1 to 4).  Examples: some drivers only ever go to lane 2, others all the way to 4.

F	Moving to faster lane.  Only two choices: 0 (move to the faster lane whenever there is an 	opening) and 1 (move to faster lane if the car in front is at the minimum following distance)

G	Minimum distance left ahead (in car-lengths).  Only move to the faster lane if the car in the faster 	lane and ahead (and moving faster) is at least G car-lengths away.

H	Minimum distance right behind (in car-lengths). Only move to the slower lane if the nearest car 	behind in the slower lane is at least H car-lengths away.

I	Moving to right lane.  The car moves to the slower lane 1= whenever there is an opportunity, 2= 	when the car directly in front is a the minimum following distance, 3= never or 4= whenever there is 	a car behind (and at the minimum following distance).

K	Same as G, only for the slower lane

L	Same as H, only for the slower lane

M	Time for lane change (in seconds)

N	Number of cars in group.  Note that, if you want to display the speed of a single car, move it into a 	separate group with the same driving parameter (groups 6 to 10 in the examples)

P	Zone ahead (in car-lengths).  Parameters G and K look at the nearest car in the lane into which 	the car wants to move.  If this car is far away, you get false results.  P creates a zone, beyond 	which all cars are ignored.

Q	Zone behind.  Same as P, but working with parameters H and L

R	Speed reporting.  Needs the symbol # followed by the number of seconds.  Thus #10 shows the 	speed of that group every 10 seconds.  If the space is left blank, the group's speed is not shown.  	Average speed of all cars is always shown.  The calculated speeds are displayed in integer mph 	(with the group number following in parentheses).

Make sure you save the altered file in .csv format.

4.  Results file.  This is also a comma-separated file (.csv), stored in the same directory as the parameter files.  You can display it with Excel and use Excel's features to display speed vs. time (highlight columns B, C, D etc. then click on INSERT, SCATTER and the diagram you prefer).